BRADFORD VTS TEACHING AIDS

CHAPTER 12

Visual Acuity & Electrodiagnostic Tests

Measuring acuity    ·    Recording reduced vision    ·    Variables affecting acuity    ·    ERG · EOG · VER

how vision is quantified, and the electrical tests that probe the retina and visual pathway

CONTENTS

A    Visual acuity — definition & measurement

B    Recording reduced & near acuity

C    Variables affecting acuity

D    Electrodiagnostic tests

Postgraduate ophthalmology teaching resource	June 2026

BRADFORD VTS TEACHING AIDS · CHAPTER 12

Visual Acuity & Electrodiagnostic Tests

Introduction

Vision has to be measured before it can be followed. This chapter sets out how visual acuity is quantified — from the Snellen chart down to perception of light — and then turns to the electrodiagnostic tests that record the electrical activity of the retina and visual pathway when imaging alone cannot explain a loss of vision.

A   Visual acuity — definition & measurement

Visual acuity is the capacity to discriminate the fine details of objects in the field of view.

The angle subtended at the eye by an object is the best measure of acuity, because it takes account of both the size of the object and the distance at which it is viewed. In practice, ‘normal’ acuity is the ability to discriminate a dimension subtending one minute of arc at the eye. A standard test letter such as ‘E’ subtends an angle of 5′ at the eye, but to identify the letter its component dimensions — which subtend 1′ — must be discriminated.

A viewing distance of six metres is adopted as standard. On the testing charts, larger test objects are supplied whose dimensions also subtend 1′ of arc, but at greater viewing distances.

WHAT “6/12” MEANS If a person has 6/12 vision, they can just respond at 6 metres to test objects that ‘normals’ can discriminate at 12 metres. The Snellen chart is used to test distance acuity in this way. If the patient wears distance glasses they should be used to read the chart, and their use recorded. If the reading is poor but improves with a pinhole, the poor acuity is usually due to a refractive error.

B   Recording reduced & near acuity

Testing children and illiterate adults

- Sheridan–Gardiner test — the patient matches Snellen letters to those on a hand-held card.

- ‘E’ test — the patient orientates a wooden E to match the orientation shown on the chart.

When the top letter cannot be seen (acuity below 6/60)

Move the patient closer to the chart. If the top letter can be read at 3 metres, the acuity is recorded as 3/60; if only at 1 metre, 1/60.

If acuity is less than 1/60, test in this descending order:

- Counting fingers (CF) at 1 metre against a white background.

- Hand movements (HM) if fingers cannot be counted.

- Perception of light (PL) — or no perception of light (NPL).

Near acuity

Near acuity is tested (with the patient’s reading glasses if needed) in good illumination at a comfortable reading distance using standard test type, and is recorded as N5, N8 and so on.

A NOTE ON MODERN CHARTS Snellen testing remains in everyday use, but logMAR (ETDRS-style) charts are now widely used in UK eye services and in research. With an equal number of letters per line and equal spacing, they give more reproducible, statistically robust measurements — useful when small changes in acuity matter, such as monitoring treatment.

C   Variables affecting acuity

1   Physical variables

- Brightness of the background.

- Illumination.

2   Physiological variables

Size of pupil. The pupil size for best acuity is about 3 mm. A larger pupil admits more light to the retina but heightens aberrations, because the peripheral lens becomes part of the light pathway; smaller pupils in bright light eliminate most ocular aberrations.

Receptor distribution. Acuity is best when the image is formed at the centre of the fovea. There are about 34,000 tightly packed cones in the foveola. As noted earlier, there are no nerve fibres, ganglion cells, bipolar cells or blood vessels at this point — just cones, their nuclei and their fibres.

D   Electrodiagnostic tests

1   The electroretinogram (ERG)

The ERG is the mass electrical response of the whole retina to a light stimulus. One electrode is attached to the eye using conductive material embedded in a contact lens, and a second electrode is placed on the forehead. The potential difference between the two is the ERG, which can be amplified and recorded on an oscilloscope.

Under dark-adapted conditions with a bright flash the waveform shows:

| Wave | Polarity | Thought to arise from |
| --- | --- | --- |
| a-wave | Negative | The receptors and retinal pigment epithelium. |
| b-wave | Positive | The layers between the receptors and ganglion cells — probably Müller’s cells. |
| c-wave | Positive | The pigment epithelium. |
| d-wave | Negative | An ‘off’ effect as the light is removed. |

Usually only the a- and b-waves can be recorded with any certainty.

Clinical applications

- To diagnose diseases causing widespread lesions of the retinal receptors and pigment epithelium — for example retinitis pigmentosa and generalised retinal disease. The response may be subnormal (reduced to about half amplitude) or extinguished (not recognisable). The ERG can detect such receptor diseases before they are clinically manifest, or before the patient has any visual complaint.

- As a valuable index of retinal function in infants, or where media opacities obscure the retina.

2   The electro-oculogram (EOG)

The EOG is an additional way of measuring retinal function electronically. Electrodes are placed at the inner and outer canthi; as the eye moves from side to side, an alternating potential is recorded. The potential difference is measured in the dark (small) and after exposure to light (large). The ratio is the light rise, a useful index of retinal pigment epithelium function.

3   The visual evoked response (VER)

Using scalp electrodes, the VER measures the electrical potential resulting from a visual stimulus. It is primarily a test of macular and optic nerve function. Abnormalities due to conditions such as demyelination can be detected, and the VER can also detect and monitor amblyopia in infants.

WHICH TEST PROBES WHICH STRUCTURE - ERG — the photoreceptors and outer retina (whole-retina response). - EOG — the retinal pigment epithelium (the light rise). - VER — the macula and optic nerve / visual pathway to the cortex.

END OF CHAPTER 12 · VISUAL ACUITY & ELECTRODIAGNOSTIC TESTS