Procedure for taking Blood Pressure

| Action | Rationale |
| --- | --- |
| 1. Explain procedure to patient | Ensure patient understanding, relax patient |
| 2. Allow patient to rest for 3 minutes if seated, 1 minute if standing | To ensure accurate reading obtained |
| 3. Ensure upper arm is supported at heart level with palm facing upward | To obtain accurate reading. Measurements made with arm dangling by the hip can be 11-12 mmHg higher than those made with arm supported. Measurements made with arm raised can be falsely high. |
| 4. Ensure that tight or restrictive clothing is removed from the arm. | To obtain accurate reading |
| 5. Use a cuff the bladder of which covers 80% of the circumference of the upper arm | To obtain accurate reading. Using cuff too small can give false high readings |
| 6. Apply the cuff snugly around the arm with the centre of the bladder covering the brachial artery. | To obtain a correct reading. |
| 7. Position Manometer at the patients heart level, but at your eye level. | Prevent tubing causing risk of accident. To obtain accurate reading |
| 8. Inflate cuff until radial pulse can no longer be felt to provide estimation of systolic pressure. Deflate cuff completely and wait 15-30 seconds before continuing. | To avoid error caused by auscultatory gap. About 5% of population have auscultatory gap and it is most common in those with hypertension. |
| 9. Inflate cuff to a pressure 30mmHg higher than the estimated systolic pressure | Pressure exerted prevents blood from flowing through the artery. |
| 10. Diagphragm of stethoscope should be placed over the point of the brachial artery with just enough pressure to keep it in place (Do not tuck stethoscope under cuff) | Excessive pressure can distort sounds or make them persist for longer than normal |
| 11. Deflate cuff at 2-3mmHg per second or per heartbeat | Slower rates can cause venous congestion and arm pain. Faster rates of deflation give inaccurate reading. |
| 12. Measurement of systolic pressure is when a minimum of two clear repetitive tapping sounds can be heard. Diastolic pressure is measured at the point where the sound can no longer be heard. |  |
| # Action | # Rationale |
| 13. A record should be made of both systolic and diastolic pressures. Compare with previous readings. Record which arm was used and position of patient. Inform appropriate personnel of any irregularities | An average of two or more readings is often taken to represent patients BP. Taking more than one blood pressure reduces the influence of anxiety. |
| 14. Remove the equipment and clean after use | To reduce the risk of cross infection. |

## Clinical Educators

Bradford Teaching Hospitals NHS Trust